DEPARTMENT OF AEROSPACE ENGINEERING, SMME, NUST, ISLAMABAD

AIRCRAFT DESIGN
Project Proposal

| Group Members | CMS ID |
| --- | --- |
| Muhammad Hayat Arain | 453834 |
| Khansa Malik | 453832 |

Corse Code: AE-424

Course Name: Aircraft Design

Submitted to: Sir Abdul-Rehman

Project Title: Conceptual Redesign of the Beechcraft Starship for Improved Payload-Range Efficiency Through Weight Reduction and Propulsion Upgrade

Design Approach: Option 1: Modification of an Existing Aircraft

## 1. Background and Motivation

The Beechcraft Starship was a revolutionary business aircraft designed by Beech Aircraft with Burt Rutan's Scaled Composites. It featured a canard-pusher configuration, an all-composite airframe, and the first certified all-glass cockpit in a business turboprop. Despite its technical innovation, the Starship was a commercial failure. Only 53 aircraft were built before production ended in 1995, and the program lost an estimated $500 million. The aircraft was considered too heavy, too slow, and too expensive compared to competing jets.

The main engineering problem was weight growth during certification. The original design target was exceeded because the FAA required conservative composite structural standards, resulting in an overbuilt airframe. Empty weight reached 10,085 lb, which limited payload and range. The PT6A-67A engines were flat-rated to 1,050 shp, further restricting performance. Cruise speed was 307 kt and range was 1,370 nmi, both below the promised values.

This project proposes a conceptual redesign of the Starship to address these shortcomings. The goal is to reduce empty weight through modern composite design and avionics, and to upgrade the engines to increase available power and efficiency. The modified aircraft should achieve better payload-range efficiency while retaining the Starship's stall resistance and low cabin noise.

## 2. Mission Requirements

The modified Starship is intended to serve as a high-efficiency business transport for the owner-pilot and corporate shuttle markets, offering jet-like speed and cabin comfort at turboprop operating costs. The primary modification objective is to improve the aircraft’s payload-range efficiency by reducing empty weight and upgrading the propulsion system.

| Requirement | Target Value |
| --- | --- |
| Primary Mission | Six- to eight-passenger business transport with 600 lb of baggage |
| Payload | 1,800 lb (816 kg) |
| Range (Full Payload) | ≥ 1,600 nmi (2,963 km) with IFR reserves |
| Cruise Speed | ≥ 340 kt (391 mph, 629 km/h) TAS at FL 350 |
| Operating Altitude | FL 350–410 (optimal cruise); service ceiling 41,000 ft |
| Endurance | 5.0–5.5 hours (mission-dependent) |
| Takeoff Distance (50 ft obstacle) | ≤ 3,800 ft at MTOW |
| Landing Distance (50 ft obstacle) | ≤ 2,500 ft |
| Occupants | 1–2 crew + 6–8 passengers |
| Other Operational Requirements | Single-pilot certification; compatibility with existing Starship ground support equipment; FAR Part 23 Commuter category compliance |

## 3. Baseline Aircraft

Aircraft Name and Type: Beechcraft Model 2000 Starship 1

Type: twin-turboprop, canard-pusher, all-composite business aircraft

Manufacturer and Background: Designed by Beech Aircraft Corporation (Wichita, Kansas) with Burt Rutan’s Scaled Composites. First flown in 1986; certified in 1988; production ended in 1995 after 53 aircraft. The Starship was intended as a replacement for the King Air 200.

Key Published Specifications:

| Parameter | Value | Source |
| --- | --- | --- |
| Empty Weight | 10,085 lb (4,574 kg) | ASAP Aerospace |
| Max Takeoff Weight | 14,900 lb (6,759 kg) | ASAP Aerospace |
| Max Ramp Weight | 15,010 lb (6,808 kg) | ASAP Aerospace |
| Fuel Capacity | 565 gal (3,785 lb) | ASAP Aerospace |
| Wing Area | 281 sq ft (26.1 m²) | ASAP Aerospace |
| Wingspan | 54.5 ft (16.6 m) | ASAP Aerospace |
| Length | 46.1 ft (14.1 m) | ASAP Aerospace |
| Powerplant | 2 × Pratt & Whitney Canada PT6A-67A, 1,200 shp each | ASAP Aerospace |
| Propellers | 5-bladed McCauley, 8 ft 8 in diameter | ASAP Aerospace |
| Cruise Speed | 307 kt (353 mph) | ASAP Aerospace |
| Max Speed | 335 kt (385 mph) | ASAP Aerospace |
| Range | 1,370 nmi (1,576 mi) | ASAP Aerospace |
| Service Ceiling | 41,000 ft | ASAP Aerospace |
| Rate of Climb | 2,748 ft/min | ASAP Aerospace |
| Stall Speed | 97 kt (flaps retracted, idle power) | ASAP Aerospace |
| Wing Loading | 53 lb/sq ft (260 kg/m²) | ASAP Aerospace |
| Power/Mass | 6.2 lb/shp | ASAP Aerospace |
| Takeoff Distance (50 ft) | 4,093 ft | Plane & Pilot |
| Landing Distance (50 ft) | 2,630 ft | Plane & Pilot |

Existing Mission and Performance Characteristics:  executive transport for six to eight passengers. The canard configuration provides excellent stall resistance. The pusher engines reduce cabin noise. The composite airframe resists fatigue and cracking. However, high empty weight limits payload and range, and the engines are flat-rated, limiting cruise performance.

Proposed Modifications:

- Structural weight reduction: The airframe was overbuilt due to 1980s composite certification requirements. Modern design methods and materials can save weight. Redesigned interior with lightweight seats, bulkheads, and panels can save 400 to 600 lb. Modern avionics can save 200 to 300 lb. A lighter landing flap system can save 200 to 300 lb. Target empty weight reduction: 1,200 to 1,500 lb, bringing empty weight to 8,600 to 8,900 lb.

- Propulsion upgrade: Replace PT6A-67A engines with modern turboprops such as the PT6A-67F or PT6E-67XP. These offer higher unflat-rated power (1,200 shp), 3 to 5 percent lower specific fuel consumption, and better diagnostics. This improves cruise speed and range.

- Aerodynamic refinements: Using iterative design methods to optimize canard incidence, wing area and wing twist to reduce trim drag. Optimize winglets to reduce induced drag.

Expected Benefits: increased payload-range efficiency, cruise speed above 340 kt, lower operating costs, and preserved stall resistance.

Reliable Sources for Baseline Data: ASAP Aerospace, Plane & Pilot, Aviastar, Simple Flying, AVweb, SAE Technical Paper 900997, Forecast International.

## 4. Preliminary Design Requirements

| Parameter | Baseline (Starship 1) | Modified Target | Justification |
| --- | --- | --- | --- |
| Empty Weight | 10,085 lb (4,574 kg) | 8,600–8,900 lb (3,900–4,037 kg) | 1,200–1,500 lb reduction from structural optimization |
| Max Takeoff Weight | 14,900 lb (6,759 kg) | 14,500 lb (6,577 kg) | Reduced empty weight allows lower MTOW for same payload |
| Max Ramp Weight | 15,010 lb (6,808 kg) | 14,600 lb (6,622 kg) | Consistent with MTOW reduction |
| Payload | Limited by weight | 1,800 lb (816 kg) | Increased payload capacity |
| Fuel Capacity | 565 gal (3,785 lb) | 565 gal (unchanged) | No structural fuel volume change |
| Wing Area | 281 sq ft (26.1 m²) | 281 sq ft (unchanged) | No planform change |
| Wingspan | 54.5 ft (16.6 m) | 54.5 ft (unchanged) | — |
| Wing Loading | 53 lb/sq ft (260 kg/m²) | ~46–48 lb/sq ft (225–234 kg/m²) | Lower weight reduces wing loading |
| Power/Mass | 6.2 lb/shp | 5.4–5.6 lb/shp | Lower weight and more available power |
| Cruise Speed | 307 kt (353 mph) | ≥ 340 kt (391 mph) | Unflat-rated engines |
| Range | 1,370 nmi (1,576 mi) | ≥ 1,600 nmi (1,841 mi) | Weight reduction + SFC improvement |
| Service Ceiling | 41,000 ft | 41,000 ft (unchanged) | — |
| Stall Speed | 97 kt | ~90–92 kt (estimated) | Lower weight reduces stall speed |
| Takeoff Distance (50 ft) | 4,093 ft | ≤ 3,800 ft | Improved power/weight and lower weight |
| Landing Distance (50 ft) | 2,630 ft | ≤ 2,500 ft | Lower weight reduces landing distance |
| Rate of Climb | 2,748 ft/min | ≥ 3,000 ft/min | Improved power/weight |

## 5. Expected Outcomes

The project will deliver a conceptual design package for the modified Starship. This will include:

- Weight and balance analysis with revised empty weight, payload, and fuel loading, and center-of-gravity verification.

- Propulsion analysis comparing baseline and upgraded engines in terms of power, specific fuel consumption, and performance.

- Aerodynamic analysis with lift and drag estimates at cruise and loiter, including effects of weight reduction and canard optimization.

- Performance analysis using the Breguet range equation for range and endurance, plus takeoff, landing, climb, and cruise calculations.

- Mission profile simulation for a typical business transport mission with fuel burn for taxi, takeoff, climb, cruise, descent, and reserves.

- Trade study on sensitivity of range and payload to empty weight, engine efficiency, and aerodynamic drag.

- Final design report documenting the process, assumptions, analyses, and comparison with original requirements and baseline aircraft.

The project will demonstrate engineering reasoning and design decision-making. Limitations and assumptions will be clearly stated.

## 6. Preliminary References

- ASAP Aerospace. Beechcraft Starship 2000 Civil Aircraft Model Details. Available at: asap-aerospace.com. (Specifications and performance data).

- Plane & Pilot. Beechcraft "Starship". 2008. (Performance data and design overview).

- Aviastar Virtual Aircraft Museum. Beech Model 2000 Starship — Executive Transport. Available at: aviastar.org. (Dimensions and performance specifications).

- Simple Flying. Curious Design: What Was The Beechcraft Starship? 2023. (Design history and market analysis).

- AVweb. Beechcraft Starship: Carbon Fiber Before Its Time. 2025. (Certification and development history).

- SAE International. Analysis and Certification of the Starship All-Composite Airframe. Technical Paper 900997, 1990. (FAA certification methodology).

- Forecast International. Beech Series 2000 Starship 1. (Weight, payload, and performance data).

- Raymer, D. P. Aircraft Design: A Conceptual Approach. AIAA Education Series. (Design methodology reference).

- Roskam, J. Airplane Design, Parts I–VIII. DARcorporation. (Sizing and performance analysis methods).

- Pratt & Whitney Canada. PT6A-67 Series Engine Specifications. (Engine performance data).

- EASA. Type Certificate Data Sheet — PT6A-67A. (Engine certification data).

- Museum of Flight. Beech Starship 1 Model 2000A. (Design and certification history).