DEPARTMENT OF AEROSPACE ENGINEERING, SMME, NUST, ISLAMABAD

AIRCRAFT DESIGN LAB
Week#03 Report

Corse Code: AE-424

Course Name: Aircraft Design Lab

Submitted by: Khansa Malik (453832)

# Thrust-to-Weight Ratio and Wing Loading Analysis

# Introduction

This study estimates thrust-to-weight ratio (T/W) and wing loading (W/S) for a jet transport comparable in size and mission to the Boeing 777-200. These parameters affect takeoff and landing distance, stall speed, climb performance, cruise range, endurance, and turning capability. Each task applies a performance requirement to estimate a design limit or an efficiency target. The results are then converted to takeoff conditions and compared to identify the most restrictive constraint.

The reference design has a takeoff weight of 530,000 lb, cruise Mach 0.84 at 35,000 ft, aspect ratio 8.68, and assumed maximum lift-to-drag ratio of 18. ISA atmospheric properties are used. The calculations are conceptual estimates and depend on the assumptions and empirical relationships selected.

# Task 1: Thrust-to-Weight Ratio

Method. Three estimates were compared: Raymer’s statistical correlation for jet transports, cruise matching, and the climb-rate requirement.

Using Mmax = 0.84, (L/D)max = 18, (L/D)cruise = 0.866(L/D)max, a weight ratio of 0.956, a thrust-lapse ratio of 0.22, and a climb rate of 1,500 ft/min gave the following results.

| Method | Takeoff T/W |
| --- | --- |
| Statistical correlation | 0.2506 |
| Cruise matching | 0.2788 |
| Climb requirement | 0.1117 |
| Selected value | 0.279 |

## Output:

### Discussion:

Cruise matching gives the highest estimate and is selected for preliminary engine sizing. The value of 0.279 is close to the cited Boeing 777-200 value of 0.283. The climb-rate estimate is lower, although the separate one-engine-inoperative climb requirement must still be checked.

# Task 2: FAR 25 and MIL-C-5011A Field-Length Checks

Takeoff and landing distances were estimated using the selected T/W, wing loading of 120 lb/ft², takeoff CLmax of 2.4, landing CLmax of 2.8, and an 8,000-ft runway. FAR 25 applies certification safety factors; the MIL-C-5011A distances use the stated demonstrated-distance definitions.

| Requirement | Estimated distance | Result for 8,000 ft |
| --- | --- | --- |
| FAR 25 takeoff | 7,703 ft | Pass; 297 ft margin |
| FAR 25 landing | 5,302 ft | Pass |
| MIL-C-5011A takeoff | 6,789 ft | Pass |
| MIL-C-5011A landing | 4,500 ft | Pass |

## Output:

Discussion

All four estimates are below the 8,000-ft runway length. FAR 25 is more restrictive because of its safety factors. The estimated FAR 25 takeoff distance has the smallest margin, so the result is sensitive to the assumptions used for thrust, lift, and field-length estimation.

# Task 3: Wing Loading from Stall Speed

At stall, lift equals weight at CLmax, giving the maximum wing loading for the selected stall-speed limit.

For Vstall = 115 kt (194.1 ft/s), sea-level density of 0.0023769 slug/ft³, and CL,max = 2.4, the calculated limit is W/S = 107.5 lb/ft².

## Output:

### Discussion

The result sets an upper limit on wing loading if the aircraft is to meet the specified stall speed. A higher wing loading would require a higher stall speed or a higher maximum lift coefficient.

# Task 4: Wing Loading from Takeoff Distance

Raymer’s takeoff parameter (TOP) relationship for a two-engine FAR 25 balanced field length was used. For the selected linear fit, L = 0.029 TOP − 0.20, with L in thousands of feet.

For an 8,500-ft balanced field length, σ = 1.0, CL,TO = 1.98, and T/W = 0.28, TOP = 300 and W/S = 166.3 lb/ft².

## Output:

### Discussion

The takeoff-distance estimate allows a higher wing loading than the stall, landing, and climb constraints reported in this study. It is therefore not the controlling wing-sizing requirement under these assumptions.

# Task 5: Wing Loading from Landing Distance

The 5,000-ft FAR 25 landing-field-length requirement was converted to a demonstrated distance using the 1.67 factor. A 1,000-ft air segment was subtracted, and the thrust-reverser factor of 0.66 was applied to the ground-roll term.

With CLmax,landing = 2.4, σ = 1.0, and Wlanding/WTO = 0.85, the reported landing-condition limit is 90.6 lb/ft², equivalent to 106.6 lb/ft² at takeoff.

## Output:

### Discussion:

The landing requirement is restrictive because the aircraft must dissipate its speed and stop within the specified distance. The reported takeoff-equivalent limit is 106.6 lb/ft², subject to the landing-distance and thrust-reverser assumptions.

# Task 6: Wing Loading for Maximum Cruise Range

For a jet aircraft at the maximum-range condition, parasite drag is taken as three times induced drag.

At 35,000 ft and Mach 0.84, the analysis uses ρ = 0.00073725 slug/ft³, V = 817.2 ft/s, q = 246.2 lb/ft², AR = 8.68, e = 0.80, and CD0 = 0.015. The reported optimum is W/S = 190.8 lb/ft².

## Output:

### Discussion:

This value is an efficiency target rather than a hard structural or operational limit. Actual cruise wing loading changes as fuel is consumed, and the optimum depends on the assumed drag polar and flight condition.

# Task 7: Wing Loading for Loiter Endurance

Maximum jet endurance is associated with the maximum lift-to-drag condition, where parasite and induced drag are equal.

The stated conditions are 30,000 ft, 180 kt, AR = 8.68, e = 0.80, CD0 = 0.015, and Wloiter/WTO = 0.85. The source report gives a code output of 135.7 lb/ft² at loiter and approximately 159.6–159.7 lb/ft² at takeoff.

## Output:

### Discussion:

The original calculations contain an unresolved discrepancy: substituting the listed density and speed into the stated equation gives a value far below the reported code output. The reported output is retained here, but this task should be recalculated from the MATLAB script before the result is used for design decisions.

# Task 8: Wing Loading from Instantaneous Turn

The required load factor is calculated from the turn rate and speed, then used to estimate the maximum wing loading.

For 18°/s at 350 kt and 15,000 ft, the report calculates n = 5.85 and combat wing loading of 44.6 lb/ft². With Wcombat/WTO = 0.85, the takeoff-equivalent value is approximately 52.4–52.5 lb/ft².

## Output:

### Discussion

This is a fighter-oriented constraint and should not size a transport aircraft. It produces a much lower wing loading than the transport requirements. The reported derived values are a 1,880-ft turn radius and a 20-s time for a 360° turn.

# Task 9: Wing Loading from Sustained Turn

Method. Sustained-turn performance is assessed by comparing available T/W with the minimum required for the requested load factor and by solving the wing-loading relation.

At Mach 0.90 and 30,000 ft, the study uses AR = 3.5, CD0 = 0.018, e = 0.65, combat T/W = 0.70, and required load factor n = 4.5. The minimum T/W is reported as 0.4517, so the condition is feasible.

| Method/result | Combat W/S | Takeoff-equivalent W/S |
| --- | --- | --- |
| Best-L/D condition (Eq. 5.22) | 28.4 lb/ft² | 33.4 lb/ft² |
| Higher root, available-thrust equation (Eq. 5.25) | 77.6 lb/ft² | 91.3 lb/ft² |
| Maximum sustained load factor | 6.97 g | — |

## Output:

### Discussion

The higher root represents the larger wing loading that can meet the specified turn requirement with the assumed available thrust. This constraint applies to fighter-type aircraft, not the transport configuration.

# Task 10: Wing Loading from Climb and Glide

The FAR 25 second-segment, two-engine-out climb requirement is represented by a 2.4% climb gradient. The high-root wing-loading solution is calculated using the available climb T/W, drag polar, and dynamic pressure.

The stated inputs are 15,000 ft, 250 kt, AR = 8.68, CD0 = 0.020, e = 0.75, climb T/W = 0.25, and G = 0.024. The report’s sensitivity output gives W/S = 66.8 lb/ft² for the 2.4% gradient.

## Output:

### Discussion

The result indicates that climb can be more restrictive than landing when the assumed climb T/W is 0.25. However, the original report also contains a conflicting hand calculation of 117.6 lb/ft². The MATLAB implementation and inputs should be checked to resolve this before final sizing. The glide case is reported as non-controlling.

# Task 11: Convert Constraints to Takeoff Conditions

Mission-condition wing-loading values are converted to takeoff weight using the mission weight fraction.

| Constraint | Mission W/S (lb/ft²) | Weight fraction | Takeoff W/S (lb/ft²) |
| --- | --- | --- | --- |
| Stall | 107.5 | 1.000 | 107.5 |
| Takeoff balanced field length | 166.3 | 1.000 | 166.3 |
| Landing | 90.6 | 0.850 | 106.6 |
| Climb, 2.4% OEI | 66.8 | 0.985 | 67.8 |
| Cruise optimum | 190.8 | 0.956 | 199.6 |
| Loiter optimum, as reported | 135.7 | 0.850 | 159.6 |
| Instantaneous turn | 44.6 | 0.850 | 52.5 |
| Sustained turn, best-L/D | 28.4 | 0.850 | 33.4 |
| Sustained turn, higher root | 77.6 | 0.850 | 91.3 |

## Output:

### Discussion

The source report gives different binding constraints depending on the configuration and assumed climb thrust. Including fighter turn constraints, instantaneous turn is the lowest value at about 52.5 lb/ft². Excluding turn constraints, the reported climb limit is about 67.8 lb/ft² when climb T/W is 0.25. If climb thrust is increased, the report states that landing becomes controlling at 106.6 lb/ft². These cases should be kept separate rather than presented as one definitive design point.

# Conclusion

The analysis estimates a takeoff T/W of 0.279 from cruise matching and compares wing-loading limits from stall, takeoff, landing, cruise, loiter, turning, and climb requirements. The field-length estimates are below the 8,000-ft runway length, although the FAR 25 takeoff estimate has limited margin. Landing gives a reported takeoff-equivalent limit of 106.6 lb/ft², while the climb result may be more restrictive under the stated T/W assumption. The final wing-sizing decision therefore depends on resolving the climb calculation and the loiter discrepancy, then confirming all constraints with a consistent MATLAB run. These are conceptual estimates and should not be treated as certification evidence.

# References

Raymer, D. P. Aircraft Design: A Conceptual Approach, 6th ed. AIAA, 2018, Chapters 3 and 5.

Roskam, J. Airplane Design, Vol. I: Preliminary Sizing of Airplanes. DARcorporation, 1985.

Federal Aviation Administration. FAR Part 25: Airworthiness Standards, Transport Category Airplanes.

U.S. Department of Defense. MIL-C-5011A: Flight Performance of Aircraft.

Boeing Commercial Airplanes. 777-200/300 Airplane Characteristics for Airport Planning, D6-58329.

Shevell, R. S. Fundamentals of Flight, 2nd ed. Prentice Hall, 1989.

# Appendix A: MATLAB Files

| File | Purpose |
| --- | --- |
| thrust_to_weight.m | Statistical, cruise-matching, and climb T/W estimates |
| check_far_mil_compliance.m | FAR 25 and MIL field-length checks |
| wing_loading_stall.m | Stall-speed constraint |
| wing_loading_takeoff.m | Takeoff balanced-field-length constraint |
| wing_loading_landing.m | Landing-distance constraint |
| wing_loading_cruise.m | Maximum-range cruise condition |
| wing_loading_loiter.m | Maximum-endurance loiter condition |
| wing_loading_instant_turn.m | Instantaneous-turn constraint |
| wing_loading_sustained_turn.m | Sustained-turn constraint |
| wing_loading_climb_glide.m | Climb-gradient and glide analysis |
| adjust_WS_to_takeoff.m | Mission-to-takeoff conversion and constraint comparison |

Note: The provided source was plain text and did not contain embedded MATLAB figures. Figure locations are marked in the report for the corresponding output plots to be inserted. The loiter and climb discrepancies are explicitly flagged rather than silently reconciled.